README:

- Use proper parentheses in your formulas or equations. For instance, 1/a-1 is NOT acceptable, as it could denote either (1/a)-1 or 1/(a-1).

- Submit your homework to Webcourse.

Q1. [15=3+4+4+4 pts]

Suppose users share a 6 Mbps link, and the total number of users is 4. Also suppose each user transmits continuously at 2 Mbps when transmitting, but each user transmits only 40 percent of the time. (See the discussion of statistical multiplexing in Section 1.3.)

a. When circuit switching is used, how many users can be supported?

b. Find the probability that a given user is transmitting.

c. Find the probability that only one of the four users is transmitting.

d. Find the probability that at any given time, all four users are transmitting simultaneously. Find the fraction of time during which the queue grows.

Q2. [15=3+4+4+4 pts]

How long does it take a packet of length 3,000 bytes to propagate over a link of distance 3,5000 km, propagation speed 2.4*108 m/s, and transmission rate 16 Mbps? More generally, how long does it take a packet of length L to propagate over a link of distance d, propagation speed s, and transmission rate R bps? Does this delay depend on packet length? Does this delay depend on transmission rate?

Q3. [15=5+5+5 pts]

Suppose Host A wants to send a large file to Host B. The path from Host A to Host B has three links, of rates R1=800 kbps, R2=4 Mbps, and R3=1 Mbps.

a. Assuming no other traffic in the network, what is the throughput for the file transfer?

b. Suppose the file is 4 million bytes. Dividing the file size by the throughput, roughly how long will it take to transfer the file to Host B?

c. Repeat (a) and (b), but now with R2 reduced to 600 kbps.

Q4. [15=5+5+5 pts]

Which layers in the Internet protocol stack does a router process? Which layers does a link-layer switch process? Which layers does a host process?

Q5. [10 pts]

Equation dend-to-end=N*L/R (Equation 1.1 in textbook) gives a formula for the end-to-end delay of sending one packet of length L over N links of transmission rate R. Generalize this formula for sending P such packets back-to-back over the N links.

Q6. [15=7+8 pts]

Consider the queuing delay in a router buffer. Let I denote traffic intensity; that is, I=L*a/R. Suppose that the queuing delay takes the form I*L/(R*(1−I)) for I<1.

a. Provide a formula for the total delay, that is, the queuing delay plus the transmission delay.

b. Plot the total delay as a function of L/R.

Q7. [15=7+8 pts]

Consider the Figure below. Suppose that each link between the server and the client has a packet loss probability p, and the packet loss probabilities for these links are independent. What is the probability that a packet (sent by the server) is successfully received by the receiver? If a packet is lost in the path from the server to the client, then the server will re-transmit the packet. On average, how many times will the server re-transmit the packet in order for the client to successfully receive the packet?